HARDWARE DESIGN DOCUMENT

8051 Microcontroller-Based Digital Clock

Version 1.0

May 2026

# 1. Project Overview

This document describes the hardware design of an 8051 microcontroller-based Digital Clock system. The system features a 16x2 LCD display for time output, three push-button inputs for user control, a buzzer for alarm or alert functionality, and a crystal oscillator for accurate timekeeping.

The design uses the standard 8051 architecture with the following key interfaces:

- 16x2 Character LCD (HD44780-compatible) on Port 2 (data) and Port 3 (control)

- Crystal oscillator (11.0592 MHz) with load capacitors on XTAL1/XTAL2

- Three user push-buttons on Port 1.0 – Port 1.2

- Active buzzer on Port 1.3

- Potentiometer on VEE for LCD contrast control

# 2. Pin Connection Summary

## 2.1 LCD Module Connections

| LCD Pin | LCD Signal | Connected To | Description |
| --- | --- | --- | --- |
| Pin 1 | VSS | GND | Ground reference |
| Pin 2 | VDD | VCC (+5V) | Supply voltage |
| Pin 3 | VEE (V0) | Potentiometer wiper | Contrast adjustment |
| Pin 4 | RS | Port 3.0 | Register Select (0=Cmd, 1=Data) |
| Pin 5 | RW | GND | Write-only mode (tied low) |
| Pin 6 | EN | Port 3.1 | Enable strobe |
| Pin 7–14 | D0 – D7 | Port 2.0 – Port 2.7 | 8-bit parallel data bus |

## 2.2 Microcontroller (8051) Connections

| 8051 Pin / Signal | Connected To | Description |
| --- | --- | --- |
| XTAL1 (Pin 19) | Crystal + Capacitor C1 | Crystal oscillator input; C1 to GND |
| XTAL2 (Pin 18) | Crystal + Capacitor C2 | Crystal oscillator output; C2 to GND |
| EA (Pin 31) | VCC (+5V) | External Access — tied high to use internal ROM |
| Port 2.0 – 2.7 | LCD D0 – D7 | Full 8-bit LCD data bus |
| Port 3.0 | LCD RS | Register Select control line |
| Port 3.1 | LCD EN | Enable/strobe control line |
| Port 1.0 | Button 1 | User input — e.g., Mode / Set |
| Port 1.1 | Button 2 | User input — e.g., Increment |
| Port 1.2 | Button 3 | User input — e.g., Confirm / Enter |
| Port 1.3 | Buzzer | Active buzzer for alarm / alert |

# 3. Component Details

## 3.1 Microcontroller — 8051

The 8051 is an 8-bit CISC microcontroller. In this design it operates from an internal ROM (EA pin tied to VCC). The clock source is an external crystal oscillator circuit connected to XTAL1 and XTAL2.

- Supply Voltage: +5V DC

- EA/VPP (Pin 31): Tied to VCC — enables execution from internal program memory

- Port 2: Configured as output for LCD 8-bit data

- Port 3.0 and 3.1: Configured as output for LCD control signals

- Port 1.0–1.2: Configured as input (with internal pull-ups) for push-button switches

- Port 1.3: Configured as output for buzzer drive

## 3.2 Crystal Oscillator Circuit

The clock circuit uses an 11.0592 MHz crystal connected between XTAL1 (Pin 19) and XTAL2 (Pin 18). Two ceramic load capacitors stabilize the oscillation.

| Component | Value | Connection | Purpose |
| --- | --- | --- | --- |
| Crystal Y1 | 11.0592 MHz | XTAL1 ↔ XTAL2 | System clock source |
| Capacitor C1 | 22 pF (ceramic) | XTAL1 to GND | Oscillator stability |
| Capacitor C2 | 22 pF (ceramic) | XTAL2 to GND | Oscillator stability |

Note: 11.0592 MHz is the preferred frequency for 8051 UART baud rate accuracy. It yields a standard machine cycle of approximately 1.085 µs.

## 3.3 16x2 LCD Module

The LCD module is an HD44780-compatible 16-column, 2-row character display. It is driven in 8-bit mode via Port 2 of the 8051.

- VSS (Pin 1): Connected to GND

- VDD (Pin 2): Connected to VCC (+5V)

- VEE / V0 (Pin 3): Connected to the wiper of a 10kΩ potentiometer for contrast adjustment

- RS (Pin 4): Connected to Port 3.0 — selects Command (LOW) or Data (HIGH) register

- RW (Pin 5): Tied to GND — sets the LCD permanently to Write mode

- EN (Pin 6): Connected to Port 3.1 — data is latched on the falling edge

- D0–D7 (Pins 7–14): Connected to Port 2.0–Port 2.7 (full 8-bit data bus)

## 3.4 Contrast Potentiometer

A 10kΩ linear potentiometer is used to set the LCD contrast voltage on the VEE pin. One end is connected to VCC, the other to GND, and the wiper goes to Pin 3 (VEE) of the LCD. Adjusting the pot changes the display contrast from fully on to fully off.

## 3.5 Push-Button Switches

Three normally-open (NO) tactile push-button switches are used for user input. Each button connects the respective port pin to GND when pressed. The 8051 internal pull-ups keep the lines HIGH when buttons are released.

| Button | Port Pin | Suggested Function |
| --- | --- | --- |
| Button 1 | Port 1.0 | Mode Select / Enter Set State |
| Button 2 | Port 1.1 | Increment (Hours / Minutes / Alarm) |
| Button 3 | Port 1.2 | Confirm / Exit Set State |

Note: Software debounce is recommended (5–20 ms delay after detecting a low logic level before confirming the press).

## 3.6 Buzzer

An active buzzer is connected to Port 1.3. An active buzzer contains an internal oscillator and sounds when DC voltage is applied. The 8051 output pin drives the buzzer directly (the 8051 can source/sink enough current for most small 5V active buzzers). If higher drive current is needed, an NPN transistor (e.g., 2N2222) can be used as a switch.

- Type: Active buzzer (5V DC)

- Control Pin: Port 1.3

- Logic: HIGH on Port 1.3 activates the buzzer

# 4. Power Supply

The system operates from a regulated +5V DC supply. All components are 5V compatible:

- 8051 Microcontroller: 4.0V – 5.5V

- 16x2 LCD Module: 4.5V – 5.5V

- Active Buzzer: 5V rated

- Push Buttons: Passive — no supply required

A 100nF decoupling capacitor placed close to the VCC/GND pins of the microcontroller is recommended to suppress high-frequency noise on the power rail.

# 5. Design Notes & Considerations

## 5.1 RW Pin Tied Low

The LCD RW pin is permanently tied to GND (write-only mode). This simplifies the interface by eliminating the busy-flag polling circuit. Instead, the firmware uses fixed time delays after each LCD command/data write.

## 5.2 EA Pin Tied High

EA (External Access) is tied to VCC, directing the 8051 to execute code from its internal flash/ROM. No external program memory is used in this design.

## 5.3 Crystal Frequency

11.0592 MHz is chosen because it divides evenly to produce standard UART baud rates (e.g., 9600 baud) and also enables accurate software timekeeping using Timer 0 or Timer 1 interrupts.

## 5.4 Port 1 Pull-ups

The 8051 Ports 1, 2, and 3 have internal pull-up resistors. When a port pin is written HIGH, the pull-up holds the line HIGH. Pressing a button pulls the pin to GND, which the firmware detects as a LOW (active-low input). No external pull-up resistors are required.

## 5.5 LCD 8-bit Mode

The LCD is driven in full 8-bit mode (D0–D7 all connected). Although 4-bit mode would free up 4 port pins, 8-bit mode provides faster data transfer and simpler firmware — both appropriate for this application given the available port pins.

# 6. Bill of Materials (BOM)

| # | Component | Value / Part | Qty | Notes |
| --- | --- | --- | --- | --- |
| 1 | Microcontroller | 8051 (AT89C51/AT89S52) | 1 | 40-pin DIP |
| 2 | LCD Display | 16x2 HD44780 | 1 | 16-pin interface |
| 3 | Crystal Oscillator | 11.0592 MHz | 1 | HC-49 package |
| 4 | Ceramic Capacitor | 22 pF | 2 | C1 and C2 for crystal |
| 5 | Potentiometer | 10 kΩ linear | 1 | LCD contrast (VEE) |
| 6 | Push Button Switch | Tactile NO switch | 3 | Buttons 1, 2, 3 |
| 7 | Active Buzzer | 5V active buzzer | 1 | Connected to Port 1.3 |
| 8 | Decoupling Capacitor | 100 nF ceramic | 1 | VCC bypass capacitor |
| 9 | Power Supply | +5V DC regulated | 1 | USB or 7805 regulator |

# 7. Schematic Connection Summary

The table below provides a concise summary of all net connections as described in the design specification:

| Net / Signal | From | To |
| --- | --- | --- |
| VCC | Power Supply (+5V) | LCD VDD, 8051 VCC, EA (Pin 31), Pot end |
| GND | Power Supply (0V) | LCD VSS, LCD RW, C1, C2, Pot end, Button common |
| LCD_D0–D7 | Port 2.0 – Port 2.7 | LCD Pin 7 – Pin 14 |
| LCD_RS | Port 3.0 | LCD Pin 4 (RS) |
| LCD_EN | Port 3.1 | LCD Pin 6 (EN) |
| LCD_VEE | Potentiometer wiper | LCD Pin 3 (VEE) |
| XTAL | Crystal Y1 | 8051 Pin 18 (XTAL2) & Pin 19 (XTAL1) |
| C1 | 8051 Pin 19 (XTAL1) | GND |
| C2 | 8051 Pin 18 (XTAL2) | GND |
| BTN1 | Port 1.0 | Button 1 → GND |
| BTN2 | Port 1.1 | Button 2 → GND |
| BTN3 | Port 1.2 | Button 3 → GND |
| BUZZER | Port 1.3 | Active Buzzer (+) → GND |

8051 Digital Clock — Hardware Design Document  |  Version 1.0  |  May 2026