User Manual

Setup Guide | Simulation | Operation

Purasthu Harini | AT89C51 Digital Clock & Stopwatch

# Prerequisites

| Software | Version | Purpose |
| --- | --- | --- |
| Keil µVision | Version 5+ | Compile firmware and generate HEX file |
| Proteus | Version 8 Pro | Load circuit and run simulation |
| Git | Any | Clone the repository |
| AT89C51 Pack | Keil Device DB | Required for MCU target in Keil |

# Getting the Code

Clone the repository from GitHub using the following command:

git clone https://github.com/varshavemula/digital-clock-stopwatch.git

cd digital-clock-stopwatch

# Build Firmware in Keil µVision

| Step | Action |
| --- | --- |
| 1 | Launch Keil µVision 5 |
| 2 | Go to Project → Open Project and select the .uvprojx file from the keil/ folder |
| 3 | Confirm the target MCU is set to AT89C51 under Project → Options for Target |
| 4 | Press F7 or go to Project → Build Target to compile the firmware |
| 5 | On successful build, a clock.hex file is generated inside keil/output/ |
| 6 | Note the full path to clock.hex — it will be needed in Proteus |

# Run Simulation in Proteus 8

| Step | Action |
| --- | --- |
| 1 | Launch Proteus 8 Professional |
| 2 | Open the simulation file: proteus/simulation.pdsprj |
| 3 | Double-click the AT89C51 component on the schematic |
| 4 | In the properties window, browse and load the clock.hex file from keil/output/ |
| 5 | Set the crystal frequency to 11.0592 MHz in the same properties window |
| 6 | Click OK to save, then press the Play (■) button at the bottom to start simulation |
| 7 | The LCD should display the current time starting from 00:00:00 |

# Operating the Device

Once the simulation is running, the LCD shows the current time in HH:MM:SS format. Use the three buttons to interact with the system:

| Button | Pin | Clock Mode | Stopwatch Mode |
| --- | --- | --- | --- |
| BTN 1 | P1.5 | Switch to Stopwatch | Switch back to Clock |
| BTN 2 | P1.6 | No action | Start / Pause stopwatch |
| BTN 3 | P1.7 | No action | Reset stopwatch to 00:00:00 |

Note: A short buzzer beep confirms every button press. The clock continues running in the background even when the stopwatch mode is active.

# Project Structure

| Path | Description |
| --- | --- |
| src/main.c | Main firmware — clock logic, ISR, LCD driver |
| keil/project.uvprojx | Keil µVision project configuration file |
| keil/output/clock.hex | Compiled HEX binary for simulation/flashing |
| proteus/simulation.pdsprj | Proteus 8 circuit and simulation file |
| docs/Project_Report.pdf | Full project overview and objectives |
| docs/Hardware_Design.pdf | Circuit schematic, BOM, and pin mapping |
| docs/User_Manual.pdf | This document — setup and operation guide |
| README.md | GitHub landing page with project summary |

# Troubleshooting

| Issue | Possible Cause | Fix |
| --- | --- | --- |
| LCD shows no text | HEX not loaded in Proteus | Re-load clock.hex in AT89C51 properties |
| LCD shows garbled text | Wrong crystal frequency | Set oscillator to 11.0592 MHz |
| Buttons not responding | Simulation not running | Click Play (■) in Proteus |
| Keil build fails | Missing AT89C51 device pack | Install pack via Pack Installer |
| Time not incrementing | Timer ISR not initialized | Check Timer 0 config in main.c |