Digital Clock & Stopwatch

AT89C51 Microcontroller | Embedded C | Proteus Simulation

Developed by Purasthu Harini

# Project Overview

This project presents a fully functional Real-Time Digital Clock and Stopwatch implemented on the AT89C51 8-bit microcontroller. The system displays live time on a LM016L 16×2 LCD and supports a dedicated stopwatch mode accessible through push buttons. The entire circuit is designed and verified using Proteus 8 simulation and the firmware is developed in Keil µVision 5 using Embedded C.

| Parameter | Detail |
| --- | --- |
| Microcontroller | AT89C51 (8051 Architecture) |
| Display | LM016L — 16×2 Character LCD |
| Development IDE | Keil µVision 5 |
| Simulation Tool | Proteus 8 Professional |
| Programming Lang | Embedded C |
| Oscillator | 11.0592 MHz Crystal |

# Objectives

- Design a real-time clock that accurately tracks and displays HH:MM:SS

- Implement a stopwatch with Start, Stop, and Reset controls

- Use Timer 0 interrupt (ISR) for precise, non-blocking timekeeping

- Interface a 16×2 LCD for clean formatted time display

- Validate the complete circuit through Proteus 8 simulation

- Provide audio feedback via a buzzer on user button interactions

# Key Features

| Feature | Description |
| --- | --- |
| Real-Time Clock | Continuously displays hours, minutes, and seconds |
| Stopwatch Mode | Independent stopwatch with centisecond precision |
| Mode Switching | Toggle between Clock and Stopwatch via BTN 1 (P1.5) |
| Start / Stop | Stopwatch controlled by BTN 2 (P1.6) |
| Reset | Stopwatch resets to 00:00:00 via BTN 3 (P1.7) |
| Buzzer Feedback | Audio confirmation on every button press |
| ISR Timekeeping | Timer 0 interrupt ensures non-blocking accuracy |
| LCD Display | Formatted time output on 16×2 character LCD |

# System Architecture

The system is built around the AT89C51 microcontroller running at 11.0592 MHz. The firmware is organized into three logical layers:

- Hardware Abstraction Layer (HAL) — LCD driver routines for command and data writing

- Timekeeping Engine — Timer 0 ISR fires every 1ms, accumulating counts for seconds, minutes, and hours

- Application Logic — Main loop polls button states, updates display mode, and formats output string

On startup, the system initializes the LCD, configures Timer 0 in 16-bit mode, enables global interrupts, and enters the main polling loop. The ISR runs independently in the background, ensuring time accuracy is never affected by display or button operations.

# Tools & Technologies

| Tool / Technology | Version / Detail | Purpose |
| --- | --- | --- |
| Keil µVision | Version 5 | Firmware development & HEX compilation |
| Proteus | Version 8 Pro | Circuit design & simulation |
| Embedded C | C89/C90 Standard | Firmware programming language |
| AT89C51 | 8051 Architecture | Main microcontroller |
| LM016L LCD | 16×2 Character | Time display output |